Keyword Protocol 2000

| OBD Service $01 |  |
| --- | --- |
| PID | Description |
| 00 | PIDs supported [01 - 20] |
| 01 | Monitor status since DTCs cleared. (Includes malfunction indicator lamp (MIL) status and number of DTCs.) |
| 03 | Fuel system status |
| 05 | Engine coolant temperature |
| 06 | Short term fuel trim-Bank 1 and Bank 3 |
| 07 | Long term fuel trim-Bank 1 and Bank 3 |
| 08 | Short term fuel trim-Bank 2 and Bank 4 |
| 09 | Long term fuel trim-Bank 2 and Bank 4 |
| 0B | Intake manifold absolute pressure |
| 0C | Engine speed |
| 0D | Vehicle speed |
| 0E | Ignition timing advance |
| 0F | Intake air temperature |
| 10 | Air flow rate from mass air flow sensor |
| 11 | Absolute throttle position sensor |
| 12 | Commanded secondary air status |
| 13 | Location of oxygen sensors |
| 14 | Bank 1 Sensor 1 |
| 18 | Bank 2 Sensor 1 |
| 1C | OBD requirements to which vehicle or engine is certified |
| 1F | Time Since Engine Start |
| 20 | Supported PIDs 21h-40h |
| 21 | Distance traveled with MIL is activated |
| 24 | Bank 1 Sensor 1 (Wide range O2S) |
| 28 | Bank 2 Sensor 1 (Wide range O2S) |
| 2E | Commanded Evaporative Purge air status |
| 33 | Barometric pressure |
| 40 | Supported PIDs 41h - 60h |
| 44 | Fuel/Air commanded equivaience ratio |
| 4D | Engine run time with MIL is activated |

| OBD Service $02 |  |
| --- | --- |
| PID | Description |
| 00 | PIDs supported [01 - 20] |
| 02 | Freeze DTC |
| 03 | Fuel system status |
| 05 | Engine coolant temperature |
| 06 | Short term fuel % trim—Bank 1 |
| 0B | Intake manifold absolute pressure |
| 0C | Engine RPM |
| 0D | Vehicle speed |
| 0E | Timing advance |
| 0F | Intake air temperature |
| 11 | Throttle position |
| 14 | Oxygen sensor voltage, |
| 20 | PIDs supported [21 - 40] |
| 33 | Barometric pressure |
| 40 | PIDs supported [41 - 60] |
| 45 | Relative Throttle Position |

| OBD Service $04 |  |  |  |
| --- | --- | --- | --- |
| Functional description |  |  |  |
| The purpose of this service is to provide a means for the external test equipment |  |  |  |
| to command ECUs to clear all emission-related diagnostic information |  |  |  |
| This includes: |  |  |  |
| • MIL and number of diagnostic trouble codes (can be read with Service $01, PID $01) |  |  |  |
| • Clear the I/M (Inspection/Maintenance) readiness bits (Service $01, PID $01 and $41) |  |  |  |
| • Confirmed diagnostic trouble codes (can be read with Service $03) |  |  |  |
| • Pending diagnostic trouble codes (can be read with Service $07) |  |  |  |
| • Diagnostic trouble code for freeze frame data (can be read with Service $02, PID $02) |  |  |  |
| • Freeze frame data (can be read with Service $02) |  |  |  |
| • Oxygen sensor test data (can be read with Service $05) |  |  |  |
| • Status of system monitoring tests (can be read with Service $01, PID $01) |  |  |  |
| • On-board monitoring test results (can be read with Service $06) |  |  |  |
| • Distance travelled while MIL is activated (can be read with Service $01, PID $21) |  |  |  |
| • Number of warm-ups since DTCs cleared (can be read with Service $01, PID $30) |  |  |  |
| • Distance travelled since DTCs cleared (can be read with Service $01, PID $31) |  |  |  |
| • Time run by the engine while MIL is activated (can be read with Service $01, PID $4D) |  |  |  |
| • Time since diagnostic trouble codes cleared (can be read with Service $01, PID $4E) |  |  |  |
| Other manufacturer-specific “clearing/resetting” actions may also occur in response to |  |  |  |
| this request message. |  |  |  |
| For safety and/or technical design reasons, some ECUs may not respond to this service |  |  |  |
| under all conditions. |  |  |  |
| All ECUs shall respond to this service request with the ignition ON and with the engine |  |  |  |
| not running. |  |  |  |
| ECUs that cannot perform this operation under other conditions, such as with the engine |  |  |  |
| running, will ignore the request with SAE J1850 May2007 and ISO 9141-2 interfaces, |  |  |  |
| or will send a negative response message with ISO 14230-4 or SAE J1979 May 2007 |  |  |  |
| interfaces, as described in ISO 14230-4 or SAE J1979 May 2007. |  |  |  |

| OBD Service $07 |  |  |  |
| --- | --- | --- | --- |
| Functional description for ISO 15765-4 or SAE J1979 May 2007 |  |  |  |
| The purpose of this service is to enable the external test equipment to obtain“pending” |  |  |  |
| diagnostic trouble codes detected during current or last completed driving cycle for |  |  |  |
| emission-related components / systems that are tested or continuously monitored during |  |  |  |
| normal driving conditions. |  |  |  |
| Service 07h is required for all DTCs and is independent of Service 03h. |  |  |  |
| The intended use of this data is to assist the service technician after a vehicle repair, and |  |  |  |
| after clearing diagnostic information, by reporting test results after a single driving cycle. |  |  |  |
| If the test failed during the driving cycle, the DTC associated with that test will be |  |  |  |
| reported. Test results reported by this service do not necessarily indicate a faulty |  |  |  |
| component / system. If test results indicate a failure after additional driving, then the |  |  |  |
| MIL will be illuminated and a DTC will be set and reported with service 03h, indicating |  |  |  |
| a faulty component / system. This service can always be used to request the results of |  |  |  |
| the latest test, independent of the setting of a DTC. |  |  |  |
| Test results for these components / systems are reported in the same format as the |  |  |  |
| DTCs in Service $03 - refer to the functional description for service $03. |  |  |  |
| If less than three (3) DTC values are reported for failed tests, the response messages |  |  |  |
| used to report the test results shall be filled with $00 to fill seven (7) data bytes. |  |  |  |
| This maintains the required fixed message length for all messages. |  |  |  |

| OBD Service $09 -PIDs supported |  |  |
| --- | --- | --- |
|  | PID | Description |
|  | 00 | PIDs supported [01 - 20] |
|  | 02 | Vehicle Identification Number |
|  | 04 | Calibration Identifications |
|  | 06 | Calibration Verification Numbers |
|  | 08 | In use Performance tracking |