Error codes

When a fault or error is detected ECU send an error code to DEUSS (column 1) and use MIL LED to report (column 3)

| Error table |  |  |  |
| --- | --- | --- | --- |
| Code | Error | MIL LED | Description |
| 10 | Err_Flash_Chk |  | Checksum error of data in flash memory The ECU usually reset the value in the default map, it could appear during a map loading interruption |
| 11 | Err_Flash_Dev |  | Flash error, it is possibile a hardware error |
| 104 | Err_Teng_CC | (0,3) 1.5 sec off 3 blinks | Engine temperature sensor in short circuit |
| 105 | Err_Teng_CA |  | Engine temperature sensor in open circuit |
| 120 | Err_Air_CC | (0,2) 1.5 sec off 2 blinks | Air valve in short circuit |
| 121 | Err_Air_CA |  | Air valve in open circuit The fault check is activated when: Air_En = 1 Output PWM air valve not 0 Engine is running |
| 130 | Err_Starter_CC | (0,5) 1.5 sec off 5 blinks | Starter in short circuit Check made if ASD starter PWM >= 5%, it works only if engine is running |
| 131 | Err_Starter_CA |  | Starter in open circuit Check made if ASD starter PWM <= 95%, it works even if engine is not running |
| 138 | Err_Speed_CC | (0,6) 1.5 sec off 6 blinks | Speed sensor in short circuit or open circuit Detected if engine is running over certain RPM and signal is fixed low or high for at least 3s Every fault conditions is immediately deleted when activity sensor is detected |
| 139 | Err_Speed_CA |  |  |
| 170 | Err_Tps_Lo | (0,1) | TPS signal lower than 0,2V |

| 171 | Err_Tps_Hi |  | TPS signal higher than 4,8V |
| --- | --- | --- | --- |
| 190 | Err_Lambda_CC | (0,4) 1.5 sec off 4 blinks | Lambda sensor short circuit to GND or 5 Volt Check if key on and engine is not running Fault detected after 5 seconds |
| 192 | Err_Lambda_NA |  | If TPS_En = 0, the conditions test are: RPM bigger than 3000 Engine run time bigger than 3 minutes If there’s no sensor activity for ClcCheckTime_NoTps seconds, the fault is activated (default value 300 seconds) If TPS_En = 1, the conditions test are: RPM bigger than 3000 Engine run time bigger than 3 minutes TPS bigger than Clc_TPSEnLo_Diag If there’s no sensor activity for ClcCheckTime_TpsPresent seconds, the fault is activated (default value 30 seconds) Sensor check for dialer: Key OFF > Key ON RPM bigger than 6000 for 1 minute within 2 minutes since 2 minutes engine’s running If there’s no sensor activity, the fault is activated Every fault conditions is immediately deleted when activity sensor is detected |